# Programming for Engineers and Scientists

# Homework 1 – Part c

Solution of a Potential Flow Problem

In the following lines it is presented and analyzed the implementation part of Finite Element code (FE) Potential Flow problem (from the previous report, Part b).

The implementation is done using the scientific software tool MATLAB.

In this case, during the Part c we are going to focus in fluid dynamics problem governed by the two-dimensional Poisson equation:

The aim in this fluid dynamic problem it is compute as a first approach the pressure flow u as a scalar field, being the main unknown of the problem. Once the pressure flow is computed, as a part of the post-process, the velocity field is obtained in terms of the potential as:

The boundary conditions and the variables considered for that problem are:

- Domain and mesh of the problem:

Figure 1: Problem Domain including mesh definition

- Material property K(x,y) it is considered  variable K(x,y) = 1 - 3.95 y2.

- The Dirichlet /Neumann boundary condition.

Figure 2: Boundary condition for the corresponding domain

*The remaining boundary is Neumann homogeneous.

# Program FE Manual:

The entire program it is contained in a folder named “Part c code”. Inside we can find different MATLAB files and structures with the extension .m and .mat (for mesh). In order to be able to run the code, the user must open in Matlab the file main.m.

The main.m contains the structure of the problem. Basically, the functions and the solution are managed in this file.

The first step to do is feed the input of the code. The program need to be provided with a mesh (nodal coordinates and connectivity matrix) and the boundary conditions (definition of inflow/outflow edge for Neumann and the nodes prescribed with Dirichlet).

This information should be stored in a structure and be located in the same folder. In our case we have already defined the input parameters on the file mesh_HW1c.mat. This folder should be loaded on the main file thought write the filename at line 19 (main.m).

Once the input file is loaded, the program procced to extract different information as Number of elements, Number of nodes per element, Total number of nodes, etc.

After read and define the variables from the mesh, the function elemType compute the required amount of gauss points, element type (triangular or quadrilateral) and the order (linear or quadratic).

Once the code is able to determine the element type and the number of gauss points required, the following function Quadrature assigns the gauss points locations z and weights w. In the same way, the function ShapeFunc give the output shape functions and its derivatives on the reference element.

## Boundary condition:

## The Neuman boundary condition starts on the code creating a structure g, where the information of inflow/outflow and the remaining boundary is stored. On the other side the function edgeElement extract and separate the element edge, saving them on the structure edge.

In order to apply the Neuman boundary condition to the correct edge for each element, the function neumnaBC assign the boundary value on the edge id (row from the structure edge). This information it is stored in another structure named ele.

## For Dirichlet boundary condition, the function dirichletBC creates a matrix C, where it is stored the value and the nodes which are imposed the bc condition.

## Stiffens matrix and force vector:

In order to create the system of equations, the first approach is creating the ensembled stiffness matrix. This action is taken by the function assembly, building the matrix of size nº degrees of freedom.

The stiffness matrix K, compute the element matrix and then do the correspondent assembly in order to build the global K.

The force vector is its computed first through a logic decision in order to compute on the elements where the Neuman bc it is different from zero. Then it is assembled in a global force vector.

The last step to solve the system is apply the Dirichlet boundary conditions and simplify the matrix K and f with the known values for the pressure. This operation it is carried on the main.m file, from line 73 until 87 arising the Ktot and f.

## Solution of the System:

In order to compute the pressure field, the total system is solved on a direct way inside the main.m file. The pressure value on each node it is stored in a column vector named as sol_u.

Post-Process:

Once the pressure value on all the elements it is obtained, the following function velField it extracts the x and y component of the velocity in each gauss element point. Also, it is required to compute the gauss points location in cartesian coordinates with the aim to plot the velocity vector field. This operation, a posteriori is assembled for each element output of the function (inside a number of elements loop).

Another important part carried as a post process are the plots, in this occasion it is evaluated as functions plotSolu (for pressure) and plotVelvector.

Finally, in order to create txt file to be read by an external software, the function vtkCreator build a file in ASIII standard with the extension .vtk ready to be displayed in PARAVIEW (visualization application).

## Interpretation of the results

In this part of the homework, we are going to analyze the results obtained after computing the code. As a reminder, we point out that the problem we have to solve in this assignment is the same equation as in Homework 1-b, with the implementation of Neumann boundary conditions and a domain which has this shape:

Figure 3: Definition of the Boundary Conditions

Another thing that we need to take into consideration is that the variable k(x) in our system has to have this value:

In order to observe the role of the variable k in our problem, we proved with different values of it obtaining different results. After some test we saw that the k variable had not a significant influence in the result (see left Figure 4). On the other hand, slight changes on the Neumann boundary condition generated significant changes in the result (see right Figure 4).

| Figure 4: Results. Left: k(x,y) = 1, using original Neumann bc. Right: Inflow = -0.7, outflow = 1, using original k variable. |  |
| --- | --- |

We applied a structured linear triangular mesh for this problem which we can see below:

Figure 5: Velocity solution plotted in vectorial field (arrows in each gauss point)

In this figure we can appreciate that the mesh consists in a uniform linear triangle mesh and all the velocity vectors plotted on every single element. In the left part (inflow) there’s not any remarkable part of the domain which should be taken into consideration, however, we can see that near the holes in the domain and more precisely near the upper and lower boundaries, the velocity vectors grow due to the reduction of space where flow is supposed to go through.

Figure 6: Pressure solution (scalar field)

If we take a look at the figure above, we can see the plot of the uniform scalar field distribution through the domain, having a 0 value in the left boundary (inflow) and 1 in the right boundary of the domain (outflow).

## Problems in Part b

Referring to the complications we had during the development of Homework 1-b we have to point the most important which was the implementation of Neumann’s boundary conditions into the code. After some time, we finally managed to include it and run the code as the problem stated.

Another issue we had in the previous assignment was plotting our model in Paraview, this was due to have a 800 node connectivity limitation for some reason, however, we successfully implemented the routine that wrote the results into a “.vtk” file every time we ran the code.

As a last point, we also had some troubles on plotting the velocity field (such as vectors on each element) in quadratic elements. The origin of this problem was that for quadratic elements, the velocity field is different for every Gauss point in every element, and the plot of our velocity field was being made in the nodes. This occurrence forces us to find another way of plotting such velocity field.

## Modifications to Homework 1-b

The modifications made for this part of the assignment were the inclusion of Neumann’s Boundary conditions, the implementation of the new domain in which we had to solve the equation and the plotting of the velocity field in the solution. As we mentioned in the previous part, this last challenge was the hardest to find a solution to, because we had to take modify the part of the code which computed the velocity and find out the way to plot the velocities; once they were computed in the Gauss points, they had to be plotted in the nodes for quadratic elements.

Figure 7: Velocity field (emphasis on the solution plotted in arrows)

In this figure we can see how the vector field is plotted on every element, in this case, we have linear triangles, thus, the arrow is placed on the center of the element.

## Conclusion

As a conclusion to this homework we have to say that this was the most challenging of the 3 parts of the homework we were supposed to submit. The reason for that is because we had to change a lot of things that we had already stablished in the previous ones. Applying Neumann’s boundary conditions was also daring.

We also had the opportunity to increase our knowledge in MATLAB and PARAVIEW.